1985 Porsche Carrera Wide Body (M491)

WPOAB0913FS121256

Adams Hudson

Requesting:

Engine out service, reseal, updates where advised.

When engine is out, owner would engine completely detailed.

- Locate and repair all fluid leaks

- Oil pressure sending unit

- Flush small oil cooler, TEST for leaks

- Oil cooler seals, o rings

- Sound pad replacement

- Check valve clearances. Adjust as needed, gaskets.

- Manifold rubber boots, check for cracks

- Manifold gaskets, check: car pops on decel sometimes

- Check vacuum lines

- Fuel filter

- CHECK Clutch and pressure plate. Advise.

- CHECK Front and rear main seals. Advise.

- Cam lines

- T-stat oil o ring (green one)

- Check crankcase breather tubes, intake rubber, hoses, torque hard to reach bolts

- Fuel lines, replace as needed

- INJECTORS – check, clean and/or replace

- Spark plugs (Bosch WR7DC <7501> if that’s your recommendation)

- Clean Air Flow meter

- Check for broken head studs when valve covers are off.

- Inspect Oil Return Tube O Rings

- Air conditioning system – check for leaks, any issues. Willing to upgrade to ‘modern’ system that incorporates different compressor, hoses, blowers, and vents. If my current system is in bad shape, would go this route.

- Shifter linkage – please repair, tighten, UPGRADE shifter to better feel and function. If 915 box can be made to shift cleanly and crisply, I’ll try it. Otherwise, and aftermarket version of your approval.

- Suspension – check all components, replace any worn components. Car ‘squeaks’ at front end.

- Brakes – same as above.

- Radio – Remove all ‘modern’ stereo components in dash, under seat. Prefer an am/fm/cd that has ‘Porsche’ on it, Ipod hookup would be nice.

- Tires – widest you can fit on factory rims in rear, OEM up front.

- Alignment for above to best ‘street’ specs.

ESTIMATE ‘range’ of labor for above

(Previous Jack Ingram bill on both engine outs/in, reseal for exact same car was $1320.)

Goal is to have all work done to completely catch the car up on services, for daily use and reliability if needed.

Any work is to be done with eye toward cleanliness and presentation, so that any old parts that get re-used are cleaned, repainted, or replated as needed.

Would like to have a thoroughly driveable, reliable car that is pristinely clean in engine compartment, underneath, suspension.