Prepurchase Inspection Form: 1973 914 2.0

Title/Registration

Name on title: __________________________________________________________________

Registered owner (if different):  ____________________________________________________

Clear (non-salvage or rebuilt) title:  _________________________________________________

Lienholder(s) listed on title: _______________________________________________________

Check VIN #’s/Paint code

On title: _______________________________________________________________

On front fender:  (Passenger side, just in front of the gas tank, above the right wheel, underneath the hood. VIN number stamped into sheet metal.)

Same as title: ____________________   Other: ________________________________

Inside front trunk:  (Near the passenger side on the side of the headlamp bucket housing.  Metal plate attached to sheet metal.)

Same as title: ____________________   Other: ________________________________

Lower left (driver side) corner of windshield:

Same as title: ____________________   Other: ________________________________

Inside driver side door jamb:  (On a sticker with the VIN number printed on it.)

Same as title: ____________________   Other: ________________________________

Paint code:  (In driver’s side front door jamb.  Shows the paint coding for the factory original color of the car.  A metal plate affixed to the door jamb.)________________________________

Engine case number:   The 2.0L engine serial number can be found on the engine case just in front of the oil filler neck.   The 2.0L engine cases always started with the letters GA or GB.

______________________________________________________________________________

Transmission case number: _______________________________________________________

Chassis/Suspension Rust Inspection

This inspection should ideally be conducted with the car on a lift for the undercarriage and suspension areas.   Also the rocker covers should be removed (six small screws) so the longitudinal chassis members and jacking points can be inspected closely.

From car underside:

The front suspension A-arm mounting points (where the suspension bolts to the chassis):

No rust ____   Very minor surface____   Minor surface ____   Serious or structural ____

Floor pan:

No rust ____   Very minor surface____   Minor surface ____   Serious or structural ____

The rocker panels - especially check the jack post support area (with covers OFF):

No rust ____   Very minor surface____   Minor surface ____   Serious or structural ____

The rear suspension mounting points, especially the right-hand side:

No rust ____   Very minor surface____   Minor surface ____   Serious or structural ____

The rear trunk pan:

No rust ____   Very minor surface____   Minor surface ____   Serious or structural ____

The floor of the front trunk:

No rust ____   Very minor surface____   Minor surface ____   Serious or structural ____

Wheel-well interiors (wheels off):

No rust ____   Very minor surface____   Minor surface ____   Serious or structural ____

Any sign of accident damage: _______________________________________________

Inside the car, lift up the carpet and look for rust around the pedal assembly, the seat mounting points, and behind the seats at the firewall:

No rust ____   Very minor surface____   Minor surface ____   Serious or structural ____

In the engine compartment-- look around the battery tray:

No rust ____   Very minor surface____   Minor surface ____   Serious or structural ____

The engine shelf below the tray. If there are big rust areas and holes here, then look under the car near at the right rear suspension console and look for structural rust there:

No rust ____   Very minor surface____   Minor surface ____   Serious or structural ____

Also in the engine compartment, look in the forward corners (use a flashlight). It is an area where water can sit:

No rust ____   Very minor surface____   Minor surface ____   Serious or structural ____

Body/Exterior

Driver’s side body panels:

Straight/no damage: ______   Note any damaged areas:________________________________

Quality of any repairs: ________   Original paint: _________  Quality of repaint: _____________

Any rust or paint bubbling areas: _________________________________________________

Comments:___________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

Passenger’s side body panels:

Straight/no damage: ______   Note any damaged areas:________________________________

Quality of any repairs: ________   Original paint: _________  Quality of repaint: _____________

Any rust or paint bubbling areas: _________________________________________________

Comments:___________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

Front trunk lid and compartment:

Straight/no damage: ______   Note any damaged areas:________________________________

Quality of any repairs: ________   Original paint: _________  Quality of repaint: _____________

Any rust or paint bubbling areas: _________________________________________________

Comments:___________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

Area between windshield and trunk lid:

Straight/no damage: ______   Note any damaged areas:________________________________

Quality of any repairs: ________   Original paint: _________  Quality of repaint: _____________

Any rust or paint bubbling areas: _________________________________________________

Comments:___________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

Rear trunk lid and compartment:

Straight/no damage: ______   Note any damaged areas:________________________________

Quality of any repairs: ________   Original paint: _________  Quality of repaint: _____________

Any rust or paint bubbling areas: _________________________________________________

Comments:___________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

Rear transverse panel:

Very straight/no damage: ______   Note any damaged areas:___________________________

Quality of any repairs: ________   Original paint: _________  Quality of repaint: ___________

Any rust or paint bubbling areas: _________________________________________________

Straight/no damage: ______   Note any damaged areas:________________________________

Quality of any repairs: ________   Original paint: _________  Quality of repaint: _____________

Any rust or paint bubbling areas: _________________________________________________

Comments:___________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

Bumpers:

Very straight/no damage: ______   Note any damaged areas:___________________________

Quality of any repairs: ________   Original paint: _________  Quality of repaint: ___________

Any rust or paint bubbling areas: _________________________________________________

Straight/no damage: ______   Note any damaged areas:________________________________

Quality of any repairs: ________   Original paint: _________  Quality of repaint: _____________

Any rust or paint bubbling areas: _________________________________________________

Comments:___________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

Brightwork around windshield:

Very straight/no damage: ______   Note any damaged areas:___________________________

Quality of any repairs: ________   Original paint: _________  Quality of repaint: ___________

Any rust or paint bubbling areas: _________________________________________________

Straight/no damage: ______   Note any damaged areas:________________________________

Quality of any repairs: ________   Original paint: _________  Quality of repaint: _____________

Any rust or paint bubbling areas: _________________________________________________

Comments:___________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

Vinyl Sail Panels/Brightwork:

Overall condition of panels: ______________________________________________________

Brightwork:  ___________________________________________________________________

Engine compartment cover:

Overall condition/operation: ______________________________________________________

Brightwork/lettering: ____________________________________________________________

Targa top:

Overall condition: ______________________________________________________________

Seals: ________________________________________________________________________

Latches: ______________________________________________________________________

Any water intrusion: ____________________________________________________________

Any problems: _________________________________________________________________

Chassis stiffness/flex test (passenger and driver’s):

Take the roof off the car.  Have someone who is heavy (200 + lbs.) to sit in the passenger’s seat (or just put weight in the seat if a big body isn’t available).   With the door closed, roll the window up. Then try to open the door. If the window sticks at the top (and does not stick with the roof on), the body is flexing.   Repeat on the driver’s side.

Passenger side flex:  ______ Yes     _____ No   	Driver’s side flex:  ______ Yes     _____ No

Interior

Check condition and cleanliness of the following and note anything that needs attention.  Rate on 1-10 scale with 10 being like new and 1 needing repair/replacement.

Dashboard: __________________________________________________________________

Center console: _______________________________________________________________

Seats: _______________________________________________________________________

Carpeting: ____________________________________________________________________

Door panels/compartments: _____________________________________________________

Headliner: ____________________________________________________________________

Rear window seal:  Intact/no rattles or looseness: ____________________________________

Check operation of and note any problem areas:

Seats and latches: ______________________________________________________________

Seatbelts: _____________________________________________________________________

Mirrors: _______________________________________________________________________

Targa top latches: _______________________________________________________________

Radio/tape/CD: _________________________________________________________________

All gauges: _____________________________________________________________________

Steering free play: ______________________________________________________________

Door/lid latches and locks: ________________________________________________________

Air conditioner (if installed): _______________________________________________________

Heater/defroster/blower: ________________________________________________________

Wipers and blades: ______________________________________________________________

Interior lights: __________________________________________________________________

Windows/windshield: ____________________________________________________________

Pedal condition: ________________________________________________________________

Brake pads:  Front % remaining: _______________   Rear % remaining: ____________________

Color of brake fluid in reseveroir: __________________________________________________

Brake lines: ____________________________________________________________________

Steering rack: __________________________________________________________________

Axle boots and seals: ____________________________________________________________

Front suspension bushings: _______________________________________________________

Sway bar intact/bushing condition: _________________________________________________

Trunks (front & rear)

Check condition and note any problem areas:

Carpets: ______________________________________________________________________

Spare tire and wheel: ____________________________________________________________

Tool kit: _______________________________________________________________________

Interior lights: __________________________________________________________________

Engine/Engine compartment

Engine lid operation: ____________________________________________________________

Missing parts/visible modifications: ________________________________________________

Oil leaks: source and extent: ______________________________________________________

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Condition of drive belts: __________________________________________________________

Condition of hoses and rubber components: _________________________________________

Overall condition and cleanliness: __________________________________________________

Cylinder compression and leak-down tests. (Note: loosen all plugs, run engine @ 3,000 RPM for 30 seconds and check each cylinder at TDC –valves closed—with a 60 PSI line pressure):

Compression		Leak-down

Cylinder # 1 		___________               ___________

Cylinder # 2 		___________               ___________

Cylinder # 3 		___________               ___________

Cylinder # 4 		___________               ___________

Exhaust system/heat exchangers: _________________________________________________

Running gear, suspension and brakes

Check condition and note any problem areas:

Wheel bearings:   Front: ___________________________   Rear: _________________________

Tie rod ends: ___________________________________________________________________

Ball joints: _____________________________________________________________________

Shocks: _______________________________________________________________________

Bushings: ______________________________________________________________________

Sway-bar/bushings:  _____________________________________________________________

Wheels—stock/optional model & sizes: _____________________________________________

Tires—brand, size and condition: ___________________________________________________

____________________________________________________________________________________________________________________________________________________________

Test Drive

Ease of starting: ________________________________________________________________

Abnormalities with clutch or gearbox: _______________________________________________

____________________________________________________________________________________________________________________________________________________________

Vibration from drive train or wheels & tires:__________________________________________

____________________________________________________________________________________________________________________________________________________________

Pulling to either side while driving or under braking: ___________________________________

____________________________________________________________________________________________________________________________________________________________

Smoothness of engine under acceleration: ___________________________________________

____________________________________________________________________________________________________________________________________________________________

Squeaks, rattles, wind noise: ______________________________________________________

____________________________________________________________________________________________________________________________________________________________

Rate overall drivability on scale of 1-10: _____________________________________________

Miscellaneous

Are owner’s manuals available or with the car? _______________________________________

Any service records with the car? __________________________________________________

How many sets of keys? __________________________________________________________

Are there any non-stock enhancements or add-ons to the car? ___________________________

______________________________________________________________________________

______________________________________________________________________________

Any other comments on the car’s condition? _________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________