1979 Porsche 928 Spyder Customization Reference Manual

Secondary Fuse Panel

|  | Circuit | Fuse | Wire Color |
| --- | --- | --- | --- |
| 12v Constant - Matte Blue |  |  |  |
| 1 | Cigarette Lighter Port & USB Sockets | 7.5 A | Yellow |
| 2 | Front Harness Feed (Front Left Fender) Driving Lights & Horn Relays |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 | Door Panel & Glove Box Lights | 5 A | Red |
| 7 | Power Door Locks | 7.5 A | Red |
| 12v Switched - Shiny Blue |  |  |  |
| 1 | Auto-Dimming Rear View Mirror with Homelink Buttons | 5 A | Yellow & Red |
| 2 | Console Accessory Gauges (Outside Temp, Boost, Air/Fuel) | 5 A | White |
| 3 | v1 Radar Detector | 2 A | Red |
| 4 | Power Feed to Console Switches (Driving Lights & Door Locks) | 5 A | Red & Black |
| 5 | Radio Power | 15 A | Red |
| 6 |  |  |  |
| 7 |  |  |  |
| Ground - Matte Black |  |  |  |
| 7 | Power Door Locks |  | 2 Black |
| 6 | Cigarette Lighter & USB Charging Ports |  | Green |
| 5 | Radio |  | Black |
| 4 | Console Switches Feed |  | Black |
| 3 | v1 Radar Detector |  | Black |
| 2 | Console Gauges (Temp, Boost, Air/Fuel) |  | Black |
| 1 | Rearview Mirror |  | Black |

Stereo Head Unit Wiring

|  | Car Side | Stereo Side |
| --- | --- | --- |
| Function | Harness | Harness |
| Ground | Black | White |
| 12v+ Constant | Red / Black Stripe | Yellow |
| 12v+ Switched (Radio On) | Red | Red |
| Reverse Signal for Camera | Dark Blue | Blue |
| Illumination / Auto-Dimming with Headlights | White | Green |
| Amp Trigger | Light Blue | Brown |

Left Front Fender (5-wire Harness from Inside Car)

- Blue – Ambient Temperature Sending Unit Signal

- White – Driving Lights (12v+ signal from switch)

- Brown – Driving Lights (Ground from switch)

- Green – Open / Available

- Yellow – Constant 12v+ from #2 on Secondary Fuse Panel

- 2nd white wire – Driver’s Side Driving Light 12v +

- Red – Passenger’s Side Driving Light 12v+

Right Front Fender Wiring

- Green – Open / Available

- Yellow – Constant 12v+ from #2 on Secondary Fuse Panel (goes to LF Fender first)

Center Console Wiring

- Blue & Green – Power Door Locks

- Red – 12v+ for Driving Light & Door Lock Switches

- Black – Ground for Driving Light & Door Lock Switches

- Brown & Green – Ground to Lighter Socket Port (Brown changes to Green)

- White & Yellow – 12v+ to Lighter Socket Port (White changes to Yellow)

Window Switch Wiring (curve is bottom right)

Brown	|	| Green

Red	|	| Black

Driving Lights Switch Wiring

12v+ Out for ON   (White)		|	|	(Black)  Ground for Switch Lights

12v+ IN from Fuse (Red)		|

12v+ Out for ON Mom (White)	|

Door Lock Switch Wiring

Lock to Lock Control (White)		|	|	Ground for Switch Lights (Black)

12v- Ground (Black)			|

Unlock to Lock Control (Brown)	|

(currently remote does not function if this switch is wired up so switch is disconnected)

HVAC Head Unit Vacuum Line Connections

Blue 	1		2  Red			3 & 11 		Jumpered together

3		4		5		4 & 14 		jumpered together

Yellow	6		7  Plugged		5 & 15 		Y together to Brown Hose

8		9		10		6		Yellow to Vacuum Switch

11		12			7		Blocked Off

13		14		15		8 & 13		Y together to Black Hose

9		goes to Blue Hose

10 & 12	Y together to White Vent Hose

Aero Mirror Wiring –

Driver’s Side (diagrams are looking into the mating side of connector)

Mirror Side Small Connector				Harness Side Small Connector

Ref	^						Ref	^

blue 	0		0   black		green / blue	0		0   purple/red

brown 	0						brown	0

red	0		0   white			brown	0		0   grey / black

white 	0						black	0

Wire Mapping – small connector

Mirror Side	to	Harness Side

Black		to	Blue / Green

Blue		to	Purple / Red

Red		to	Grey / Black

White		to	Black

Brown		to	Brown

Brown		to	Brown

Passenger’s Side (diagrams are looking into the mating side of connector)

Mirror Side Small Connector				Harness Side Small Connector

Ref	^						Ref	^

red 	0		0   blue		purple / red	0		0   grey / black

brown 	0						brown	0

white	0		0   brown		brown		0		0   black

black 	0					blue / green	0

Wire Mapping – small connector

Mirror Side	to	Harness Side

Black		to	Blue / Green

Blue		to	Purple / Red

Red		to	Grey / Black

White		to	Black

Brown		to	Brown

Brown		to	Brown

Wire Mapping – large connector

Car Side		to	Harness Side

Green / White		to	Green / White

Black			to	Black / Grey

Blue / Green		to	Black

White		 	to	White

Brown			to	Brown