Porsche 928 Front Suspension Torque Values

Upper Control Arm to Body – 103 ft.lbs.

Lower Control Arm to Body – Front – 62 ft.lbs.

Lower Control Arm to Body – Rear – 88 ft.lbs.

End Nuts of Upper Control Arm for Rubber Bushings – 62 ft.lbs. (torque w/ suspension loaded)

Front Lower Control Arm Clamp Bolts – 62 ft.lbs (torque w/ suspension loaded)

Rear Lower Control Arm Clamp Bolts – 88 ft.lbs (torque w/ suspension loaded)

Sway Bar Drop Links Nut/Bolt – 62 ft.lbs.

Wheel Bearing Clamping Castle Nut Allen Bolt – 11 ft.lbs.

Ball Joint to Lower Control Arm (Camber & Caster Eccentric nuts) – 88 ft.lbs.

Steering Track Rod to Steering Arm – 47 ft.lbs.

Brake Caliper to Steering Knuckle – 62 ft.lbs.

Front Lower Control Arm Guard Mounting Bolts – 33 ft.lbs.

Lower Strut Clamp to Body Mount – 62 ft.lbs.