### Uncorking The Vin

Like the vintage of a great bottle of wine the VIN of your 928 can tell you something of its pedigree. But how do you decode that string of numbers & letters into something with more meaning? What can it tell you about your car anyway? Lets find out…

# The VIN Standard

The standardized 17 character Vehicle Identification Number (VIN) was introduced in the USA in 1980 although its use only became mandatory for 1981. Porsche thus adopted the VIN standard in model year 1981. Before this manufacturers including Porsche chose their own serial numbering schemes. 928’s before the 1981 model year used a scheme with just 10 characters usually starting with 928. Unfortunately the remaining characters in the early numbers do not convey much information beyond year, engine type & serial number.

Fig 1. Pre-VIN 928 Serial Numbers:

| Year | Prefix | Engine/Year | Serial # |
| --- | --- | --- | --- |
| 1977 | 928 | 820 | #### |
| 1978 | 928 | 820 | #### |
| 1979 | 928 | 920 | #### |
| 1980 | 92A0 | 81 | #### |

Note only serial numbers distinguish 1977 & 1978 models (but 1977 was very low volume in the US). Euro/ROW models have different Engine/Year codes e.g. 810 for 1978, presumably due to emissions related differences.

Today many manufacturers, including Porsche, deploy a 17 character vehicle ID system ‘similar’ to the USA VIN across their worldwide supply - probably to simplify their vehicle ID marking requirements. The European Union requires cars to comply with ISO 3779 defining a 17 character ID, which is essentially a sub-set of the USA VINs.

# Anatomy of a Vehicle Identification Number

The 17 character USA VIN complies with - and extends upon - the ISO 3779 standard comprising 3 component sub-sections.

Fig 2.

Generic VIN Component Parts (based on ISO 3779 spec definitions)

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WMI |  |  | VDS |  |  |  |  |  | VIS |  |  |  |  |  |  |  |

Digits 1-3     	=   WMI	- Worldwide Manufacturer Identifier

Digits 4-9     	=   VDS	- Vehicle Descriptor Section

Digits 10-17  	=   VIS  	- Vehicle Identifier Section

The WMI is used to uniquely identify all significant volume manufacturers worldwide, all Porsche VIN’s start with the characters “WP0”. Despite appearances and the apparently obvious relationship, this is actually a ‘W’ a ‘P’ and a Zero character. (in fact the letters I, O and Q are never used in a VIN because they are too easily confused with numbers or other characters).

The VDS contains information about the vehicle some of which is standard while some is manufacturer specific. For a 928 this includes model information, safety equipment information, engine & body type information. The last character of the VDS section is a checksum character that is generated to validate the whole VIN. The purpose of this is to ensure that single character errors can be detected – this can be used to ensure that most transcription or reading errors can be detected. It’s relatively easy to generate the checksum character from the other characters & then to validate the VIN. This is how CARFAX etc services initially validate a VIN number you give them.

The VIS section is used - in conjunction with the WMI & VDS - to uniquely identify a single car. It incorporates a unique series production number, the manufacturing location & model year. For high volume manufacturers this of course requires many character positions for uniqueness. The VIN standard allows for just less than 1 million cars per model/type per location per year, likely more than adequate. Porsche, being a relatively low volume manufacturer, has adopted for the 928 some sub-coding within the VIS that provides further model information, but limits the total vehicles that can be uniquely defined to just less than 10,000 per model/type/location/year. This is quite workable since the 928 in its highest volume year only approached half this number in worldwide sales.

Insurance companies use the VIN to validate vehicle year, types, engines & equipment against their rate structures. You can use it to ensure you understand the exact model of your car and that major items are still as originally equipped. Of course the 928 option codes can tell you a lot more about other installed equipment.

# Breaking It Down

Lets take a typical 928 and decode its VIN. I’ll choose a 1986 928S as the largest volume 928 ever sold in the USA*. Since I don’t have one I looked in the Auto Trader and found a suitable VIN donor car: WP0JB0920GS861678. Lets decode this one…

* ~2780 US cars sold ~57% of worldwide production (source: 928 Specialists web site).

Fig 3. Handy VIN Decoder Table:

Describing each character position for our example & generically for 928s.

| ## A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- |
| Char Pos’n | Char Type* | Description | VINStd? | ## Target 928 VIN Example | Notes (& other Porsche / etc usage) |
| 1 | X | Country Of Manufacture | Std | W = Germany | 1 or 4 = USA, 2=Canada, 3=Mexico, J=Japan, Z=Italy, S=England… |
| 2-3 | XX | Manufacturers ID | Std | P0 = Porsche | Note a “P” & a ‘Zero’ AU = Audi, Fx = Ford, VW = Volkswagen, 1x=Chevy, Cx=Chrysler… |
| 4 | A | Body Type | P | J = 2 Door Coupe | J or A = 2 door coupe. Z =>Euro/ROW J was used before 1991, A used in 1991 & later. (For 911’s: E &B = Targa, C = Cabrio…) |
| 5 | A | Engine Type | P | B = Gasoline | All 928’s either A or B, both => gasoline engine B used only during period 83-’90, Z =>Euro/ROW |
| 6 | X (~#) | Safety Restraint Systems | Std | 0 = Manual Seat Belts | 2 = Man. Seat Belts + Dual Airbag, Z =>Euro/ROW |
| 7-8 | ## | Model ID (first part) | P | 92 =1st 2 digits of 928 | Always 92 for all 928’s (96 = 968, 93= 930) |
| 9 | X (~#) | Checksum Character | Std | 0 = Used to help check validity of VIN | 0-9 or X=10, Z =>Euro/ROW Character computed based on the assigned character values of characters 1-8 & 10-17 multiplied by position dependant weighting factors. The sum of products is divided by 11, this remainder is the check digit number. See more details below |
| 10 | X | Model Year | Std | G = 1986 | A=1980, B=81, C=82, D=83...H=87, J=88...M=91, N=92, P=93, R=94, S=95, T=96, V=97...Y=00... (Chars I, O, Q, U, Z, 0 not used for clarity - skipped) Model years run from Sept 1st of previous year to Aug 31st of the nominal model year. Note build date can be earlier & sales/in-service date later than this. (Porsche started using VINs in ’81, no US 92 model) |
| 11 | X | Assembly Location | Std | S = Stuttgart (Zuffenhausen factory), | Place the VIN is affixed. Always = S for all 928’s V=Valmet, Finland (e.g. for Boxsters) |
| 12 | # | Model ID (last part) | P | 8 = the last digit of 928 | Always = 8 for all 928’s |
| 13 | # | Body/Engine Type Code | P | 6 = USA (83-90) S/S4/GT | 0 = Euro/ROW GT/GTS (91-95) 1 = USA GT/S4 (91) NB: no ’92 US imports 2 = USA 928 (81-82) & later: GTS (93-95) 4 = Euro/ROW S/S2/S4 (81-90) 6 = USA S/S4/GT (83-90) |
| 14-17 | #### | Production Serial Number | Std | 1678 = Unique ID# | Lower  Earlier, but may not start at 0 NB: some # blocks reserved for special editions |

Fig 4.  Column Explanations for Fig 3.

| Column Above | What it Represents & Any Coding Used |
| --- | --- |
| A | Character Position(s)# in the VIN starting with 1 at the left |
| B | Type of Character: A = Alphabetic (Letter) Only X = Either Alphabetic or Numeric # = Numeric (0-9) Only |
| C | Description – meaning of this/these character(s) |
| D | Standard VIN category/usage or Porsche specific usage Std = Standard VIN Terminology (as defined in VIN Std or by de-facto usage) P = Porsche Specific Category/Terminology |
| E | Character Example for the target VIN & explanation |
| F | Other 928 Options, valid content, other Porsche / etc usage |

# The Checksum Character:

The checksum is calculated last after all the other fields of the VIN are defined. The contents of each character field is converted to a value using the look-up table of Fig 5. Then each character’s value is multiplied by a weighting factor for that character position from Fig 6. The product terms for positions 1-8 & 10-17 are then summed and divided by 11 and the numeric remainder becomes the checksum character. If the remainder value is 10 the checksum character is “X”.

# Fig 5. The Character Value Translation Table

| VIN Char. | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Char. Value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| VIN Char. | H | J | K | L | M | N | P | R | S | T | U | V | W | X | Y | Z |  |
| Char. Value | 8 | 1 | 2 | 3 | 4 | 5 | 7 | 9 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |

Fig 6. The Character Position Weighting Factor

| VIN Char# | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Weighting | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 10 |  | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 |

Here is a handy way to represent & perform the calculation in a grid format using our VIN ‘donor car’ as the example.

Fig 7. Checksum Grid   (also easily implemented in a spreadsheet)

| Char. Position | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Your VIN | W | P | 0 | J | B | 0 | 9 | 2 | 0 | G | S | 8 | 6 | 1 | 6 | 7 | 8 |
| Char. Value | 6 | 7 | 0 | 1 | 2 | 0 | 9 | 2 | 0 | 7 | 2 | 8 | 6 | 1 | 6 | 7 | 8 |
| Weighting | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 10 | ^ | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 |
| Product | 48 | 49 | 0 | 5 | 8 | 0 | 18 | 20 | 385 | 63 | 16 | 56 | 36 | 5 | 24 | 21 | 16 |
| Sum of Products -----------------------------------------------^ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

385 divided by 11 = exactly 35 (remainder is 0) - The VIN matches this so is valid.

Note: Euro/ROW cars have a ‘VIN like’ number that includes the character “Z” in positions 4-6 & 9. There is no checksum and no Body, Engine or safety equipment coding. This explains why import’s VINs often can’t be validated (e.g. by CARFAX) and may create issues with insurance companies. Some may have had a substitute VIN#’s issued on import for this purpose.

# Uses of your VIN

# For a fee you can use your VIN to track build details, applicable recall notices, warranty claims, thefts & insurance claims as well as registration & title changes.

# OK so where is it

So you know how to decode it, but how do you find it – well of course most of us have seen the VIN plate in the windshield area attached to the drivers side A-Pillar, but the VIN should also appear in the following easy to find locations and all should match:

- Original guarantee & maintenance booklet, also other original vehicle sale documents including the window sticker.

- Title & state registration documents

- Emissions test compliance documents

- Metal plate riveted to drivers side A Pillar

- Under hood on inside lip of passenger (R) fender

- Sticker on trunk/hatch floor under carpet above gas tank (also option codes here)

- Sticker on drivers door post

- For Euro only: Underside of hood near release lever

When checking the VIN at these locations look for signs of any tampering, all should be clearly legible and should pass the checksum test. Be extremely suspicious if any VIN plates have been modified or removed, look for other proof of authenticity. One option is to get a CARFAX and match the results against any known vehicle history facts. It’s also possible to request a “Kardex” or Certificate of Authenticity from PCNA costing about  $85. This certificate confirms car build details like: paint color, interior finish materials, equipment including options and engine/ transmission type/serial numbers. It may also detail sticker price and completed warranty/recall work, but it requires proof of ownership and takes considerable time so is more appropriate for an existing owner than to evaluate a potential purchase. You may have some luck getting some basic details by phone from PCNA providing you can get the owners approval & support. Search on the web under Kardex for more details.

# There you have it:

You should now understand what data is encoded into the VIN number and be able to decode this information for any 928. Hopefully I’ve also demystified the magical checksum character and how you can use it to validate a VIN yourself.