phatblackie build 2010/11

Motor:

3.8l twin plug TT built by Andial. Thielert and fvd ecu’s. Fabspeed headers. AWE test pipes. Heat exchanger, cat, muffler and AC compressor delete.

Transmission:

Complete AWD delete.  GT2 G50 transmission with steel synchro’s. Third, forth, fifth and sixth re-geared for Monticello. RS shifter and Guard LSD. Tilton 5.5-inch 3-disk clutch.

Suspension:

700# front and 900# rear springs. Moton Clubsport dampers with camber plates. PMS font and rear RSR uprights/center lock spindles/racing bearings /rear uniball track links and sway bar links (ex-Penske Racing). PMS 24mm front and 20mm rear sway bars. Cup strut tower brace.

Brakes:

GT2 Evo calipers with 380mm AP front rotors and 350mm rears. Both slotted with Wilwood hats and Pagid pads.

Body:

OEM sunroof and headliner delete roof. OEM Euro NA front and TT rear bumpers. Getty front splitter. PMS fender flares. PMS rear wing with Club Sport risers fabricated by DeMan Motorsport. Euro taillights. PMS aluminum hood and doors (doors ex-Rohr Racing). Headlight washer, fog light, air intake scoop, rear wiper and third taillight delete. BBS MS 10x18 and 12x18 aluminum wheels. Hoosiers - 245 front and 315 rear.

Interior:

Full cage fabricated by DeMan Motorsport. Recaro seats. RS door cards with wind-up windows. GT2 instruments. Non-airbag QR steering wheel. Passenger side airbag, front and rear speaker, carpet, soundproofing, radio and rear seat delete. Trunk mounted carpet, windshield washer, CD player, spare tire, jack and toolkit delete. AC condenser and evaporator delete.

Weight:

2890 lbs with all fluids and 1/2 tank of fuel.  This is a work in progress.