Michelin's Monte Carlo range

| #### Prevailing conditions | ###### Type of tyre theoretically the most suited | Dimension | Technical description |
| --- | --- | --- | --- |
| Stages with heavy snow cover (70%-100%) and/or ice. | Michelin G51 (Studded) | 10/65x16 | Maxi snow pattern and construction. This tyre's narrow dimension (100 mm) enables it to cut through the upper crust of snow with a view to seeking out better grip deeper down. Its high sidewalls offer good shock absorbing performance and positive steering response on snow-free roads. |
| ##### Mixed stages (ice/ snow and clear asphalt). Snow cover between 30% and 70%. | Michelin D10 or Michelin D10 Evolution Number of studs can be adapted according to prevailing conditions. Also available without studs. | 18/65x18 | Available in two versions (base tyre + evolution), the pattern and construction of the Michelin D10 make it suitable for running on clear tarmac. Wider than the Michelin G51, it ensures a bigger contact patch for more efficient transmission of the car's power to the ground. On dry and damp surfaces, its low sidewalls favour lateral stability under cornering. |
| Dry tarmac (ground temperature: up to 8°C) | Michelin N0 or Michelin N0 Evolution 1 | 20/65x18 | These are all racing tyres, although their individual patterns, compounds and constructions differ according to their end use and conditions (dry weather, damp surfaces, rain, temperature, etc.). Their tread width is the maximum authorised by the regulations (9", or 228mm) in order to optimise the transmission of the car's power to the ground. Like the Michelin D10, their low sidewall dimensions favour cornering stability. |
| Dry tarmac (ground temperature: from 6°C) | Michelin N2 or Michelin N2 Evolution 1 or Michelin N2 Evolution 2 | 20/65x18 |  |
| Damp tarmac | Michelin TA0 | 20/65x18 |  |

- Michelin has a total staff of 30 in Monaco. The total number of covers available for Michelin's WRC partners is 3,860 carried in 10 semi-trailer workshops. All tyres are fitted with Michelin's ATS system (Appui Temporaire Souple). These figures do not include those teams supplied and serviced by Michelin France.

- The tread patterns of the N and the TA can be fine-tuned to match the conditions on the day thanks to the technique of re-cutting.

- (*) Width (cm) / exterior diameter (cm) x interior diameter (inches)