| Vehicle Height | Workshop Manual | Roland Kussmaul |
| --- | --- | --- |
| Front axle height | 115 + 5 mm (115 to 120 mm) | same |
| Rear axle height | 128 + 5 mm (128 to 133 mm) | same |
| Wheel Alignment |  |  |
| Front Axle |  |  |
| Toe unpressed total | +8' ± 2' | +5’ |
| Toe difference angle at 20° lock | -1° 30' ± 30' | -1° 20’ ± 30’ |
| Camber (with wheels straight ahead) | -1° ± 5' | -2.5° ± 5' |
| Max. difference, left to right | 10' | 5’ |
| Caster | 8° ± 30' | same |
| Max. difference, left to right | 40' | 30’ |
| Rear Axle |  |  |
| Toe per wheel | +13' ± 2' | +17’ ± 2' |
| Max. difference, left to right | 5' | same |
| Camber | -1° 50' ± 5' | -2.3° ± 5' |
| Max. difference, left to right | 10' | same |