| Competition Regulations |  | 2010 |
| --- | --- | --- |
|  | Version 3.0 (final) |  |

All contents of this document are copyright ©2009-2010, CaymanSpec Racing, all rights reserved.  CaymanSpec, CaymanSpec Racing, 987Spec, and 987Spec Racing are copyright ©2009-2010.

CaymanSpec Racing is not affiliated with or supported in any way by Porsche AG, PCNA, PMNA, PCA, or the POC.

Contents

Section 1 – Purpose of CaymanSpec Regulations	2

Section 2 – Definitions	2

Section 3 – Eligibility	2

Section 4 – General Rules	2

Section 5 – Safety	3

Section 6 – Allowable Modifications	3

Section 7 - Weight	7

Section 8 - Revisions	7

# Section 1 – Purpose of CaymanSpec Regulations

The letter and spirit of these regulations are intended to govern closed course racing among standard Porsche Cayman S cars, encourage fair and even competition, further driver development, control both build out and running costs, and set the expectation of honorable behavior among participants.

Proper application of these rules is intended to create a race class that is broadly applicable among multiple sanctioning bodies (e.g., Porsche Owners Club, Porsche Club of America, etc) where driver skill and car setup determine both individual race and annual championship winners.

Safety, reliability, consistency, and increasing car count will govern the continual development of these regulations. Speed and comparative performance to other classes are secondary considerations.

To further the applicability of this race class these regulations are intentionally similar in nature to other production-based racing series rule sets. Pending acceptance by the sanctioning bodies, the class will tentatively be referred to as CaymanSpec or Cayman Spec Racer using the class designation CSR.

# Section 2 – Definitions

2.1 	POC - Porsche Owners Club

2.2	PCA - Porsche Club of America

# Section 3 – Eligibility

Initial eligibility is limited to model year 2006, 2007, and 2008 3.4 liter Cayman S USA specification cars that are properly modified for racing (per CaymanSpec regulations).  Integration of other Porsche model 987 production cars may be considered (e.g., 987 Gen II Cayman S with TBD weight penalty), 987 Gen I or Gen II Boxster S).

Eligible “donor” cars must have been sold for street use, with all standard parts, and not produced solely for racing purposes.

# Section 4 – General Rules

All modifications must be governed by either these regulations or by the rules of the sanctioning body.  In the event of conflict the rules of the sanctioning body rules will govern (e.g., safety equipment). Proper class designation (CSR) must be visible on the car per sanctioning body’s rules.

No change may be made to a standard car that is not explicitly allowed by these rules.  There is no “grey area”.  If it is not called out in either the general rules or in specific sections the change is not allowed, period, without the permission of the CaymanSpec rules team (representation to be defined).

Changes made to the car without the knowledge of the driver are not a defense for disqualification.

# Section 5 – Safety

Sponsoring organization (e.g., POC, PCA) regulations will govern all safety requirements.

# Section 6 – Allowable Modifications

6.1 	All modifications, additions, or deletions explicitly mentioned in the sub-sections below are allowed, but may not be required.  Stock components are always allowed.  Where changes are not explicitly mentioned all components of the reference system must be kept stock.

6.2	Fuel System

6.2.1	OEM fuel tank or approved fuel cell is required – fuel filler may be relocated to trunk

6.2.2	Total fuel capacity must not exceed OEM specification of 16.9 US gallons

6.2.3	Any commercially available gasoline up to and including 100 octane

6.2.4	All OEM fuel system emission control/recovery systems must remain in place when the stock fuel tank is maintained

6.3	Brakes

6.3.1	Emergency brake system and all associated components may be removed

6.3.2	Dust shields may be removed

6.3.3	Steel braided brake lines may be installed

6.3.4	Brake pads are open

6.3.5	Brake rotors may be replaced with any one piece rotor of the same dimension as stock rotor including solid, drilled, or slotted rotors (no two piece rotors)

6.3.6	Ducting of air through the front valance to the brakes is permitted.  Fog lights may be removed to facilitate placement of air ducts

6.4	Suspension

6.4.1	OEM PASM system may be removed

6.4.2	OEM Struts may be replaced with any single or double adjustable damper

6.4.3	Any spring rate may be used

6.4.4	Strut top mounts may be replaced with GT3 Street or Cup mounts

6.4.5	Strut braces are permitted

6.4.6	Installation of the 996 GT3 “Street” control arm components are permitted both front and rear – no solid mounts (PN 996 341 121 90, 996 341 122 90, 996 341 441 90, or supersede, plus shims and hardware)

6.4.7	Adjustable rear “toe links” are permitted (bump adjustment OK)

6.4.8	Front sway bar must be stock or OEM 997 GT3 (PN 997 343 701 91 or supersede)

6.4.9	Rear sway bar must be stock or Tarett Engineering (19.1 mm OD)

6.4.10	Drop links and drop link mounting locations are open

6.4.11	Minimum ride height is 3.5 inches measured anywhere on the car

6.4.12	Suspension pickup points must remain stock (except drop link mount points)

6.5	Wheels

6.5.1	Front: 18 x 9 max; Rear: 18 x 10 max

6.5.2	Wheel spacers are allowed up to 15 mm thickness

6.5.3	Conversion to studs is allowed

6.6	Tires

6.6.1	Any DOT R tire is permitted (e.g., Hoosier R6, Kumho V710).  One set per event (two day race weekend).  Enduro events (1 hour or greater) are excluded.

6.7	Power Steering

6.7.1	Any reliability oriented modification to the OEM power steering system is allowed, including the replacement of the fluid lines and fittings, replacement and/or relocation of the fluid reservoir and installation of a remote fluid cooler

6.7.2	Electric power steering system may be permitted (need POC ruling)

6.8	Engine – M97.21 (3.4L)

6.8.1	Blueprinting and balancing of the engine is allowed – no modification to the lightest of any multiple components (e.g., lightest piston) ; pistons, bore, stroke, and compression ratio must remain stock ; no modification to the cylinder heads other than machining to bring compression ratio to OEM specification

6.8.2	Any reliability oriented modification to the to the engine is allowed including machining of the OEM crank to improve oiling, installation of high strength connecting rods, installation of an oil sump extension, and modification to the oil de-aeration system

6.8.3	Installation of an Accusump system and control valve is permitted

6.8.4	Installation of the improved “motorsports” twin chamber Vapor Oil Separator (VOS) is allowed (PN 996 107 926 00 or supersede)

6.8.5	Installation of external oil cooler is permitted

6.8.6	Intake to remain stock (including plenum and manifold).  Filter element is open and “snorkel” plate may be removed)

6.8.7	Exhaust is free (catalysts may be removed and headers installed)

6.8.8	Underdrive crank pulley is allowed

6.8.9	Coolant – Replacement of Ethylene Glycol is required.  Additives such as Redline Water Wetter are permitted

6.8.10	Addition of the OEM center radiator is permitted

6.8.11	Modification to the OEM DME is permitted (e.g., flashing)

6.8.12	Air conditioner, air pump, cruise control and accessory components may be removed

6.9	Driveline

6.9.1	TBD or LSD is permitted

6.9.2	Lightweight flywheel is permitted

6.9.3	Performance clutch is permitted (maintain OEM dimension)

6.9.4	Transmission may be re-built, but must maintain stock surfaces and gear ratios

6.9.5	Short shift kit is permitted

6.9.6	External transmission cooler is permitted

6.10	Body

6.10.1	Removal of the forward side windows and window mechanism is permitted

6.10.2	Removal of the door internal bracing is permitted if required by roll cage installation

6.10.3	Front and rear bumper may be modified to allow for the installation of tow hooks

6.10.4	Rear bumper center section may be cut out (area recessed for license plate) to allow for improved engine and transmission cooling

6.10.5	Protective screen material may be installed in front of the radiators and engine intake

6.10.6	Battery must remain in stock position (lightweight battery OK)

6.10.7	OEM rear wing may be replaced with any wing having maximum dimensions of 9.75 inches x 60 inches.  Wing may not extend above the roofline or beyond the rear bumper.  OEM wing mechanism may be removed

6.10.8	Front splitters may not extend forward of the front bumper

6.11	Interior

6.11.1	Passenger seat may be removed

6.11.2	All interior trim, with the exception of the upper portion of the dash, may be removed

6.11.3	Dash may be modified to allow for installation of roll cage

6.11.4	Airbags may be removed

6.11.5	Steering wheel may be replaced with a race-type steering wheel

6.11.6	Steering wheel quick release is permitted

6.11.7	HVAC and audio systems may be removed

6.11.8	Driver service items may be installed (e.g., mirrors, cool suit, radios, cameras, etc.)

# Section 7 - Weight

7.1	Minimum weight including driver is 2850 pounds (1313 Kg)

7.2	No more than 66 pounds (30 kg) of ballast may be added to make weight

7.3	Any ballast must be properly contained and bolted to floor in the passenger seat well

# Section 8 - Revisions

7.1	Suggested revisions to the rules will be considered by the CaymanSpec rules team

7.2	Periodic (during the season) rules revision may be necessary to correct errors

7.3	Annual revisions will be effective January 1st.  Suggested rules revisions must be submitted to caymanspec@cox.net no later than July 1st for consideration by the rules team.  Preliminary comments will be provided by October 1st and final revisions will be published by November 1st